Hallowell H-Post High Capacity Closed Shelving Specifications

Shelving shall be GREENGUARD Gold CertificationSM

H-Posts:  Fabricated from 14 gauge cold rolled steel and roll formed into an 1-1/2" wide x 2-15/16" deep tubular section.  Rear flanges are welded on 6" centers to provide maximum strength.  Sides of the post are punched 1-1/2" on center with 3/8" x 7/8" rectangular slots to receive shelf clip.  Rear flanges of the post have holes punched on 3" centers for the attachment of side sway braces or field attached solid side panels.  Post front is slotted on 3” centers for beam connection and combination clip attachment and is punched on 3” centers for field attached back connector strips. Provide (2) H-Posts between each adjoining section and at row ends.

Back Panels:  Fabricated from 18 gauge cold rolled sheet steel.  The two outer edges and the center shall be punched with 5/16" diameter holes on 1" centers running vertically for assembling to back posts with connector strips.  The center row is used to attach to rear of shelf using Back Clips.  Provide at back of closed-type shelving sections.

Side Panels:  Fabricated from 18 gauge cold rolled sheet steel.  Multiple panels are field bolted to front and back posts.  Provide at all side locations for closed shelving sections.

Foot Plates:  2-1/4" x 3-1/2" x 11 gauge steel plate with mounting bracket welded on top.  The bracket is punched for fastening to H- posts.  The plate shall have a 7/16" diameter hole punched on one end for floor anchoring and be embossed on the other end for Shim Plate alignment.  Provide as required for floor anchoring or when shimming is required for a level installation.

Shelves –Heavy Duty:  Formed with two ribs along the horizontal front and rear edge and are stamped on the top left corner for identification.  Class 1 shelves are fabricated from heavy gauge cold rolled steel and have roll formed rectangular tubular shaped front and rear edges with return bends resistance welded to the under side of shelf.  Welds are spaced 3"on center to provide maximum strength.  Sides are triple flanged to form a channel.  All four corners are lapped and resistance welded to provide a rigid corner and add extra strength to the shelf.  Top surface shall be punched for attaching Shelf Dividers from left to right on 1-1/2" centers, front and back, for 36" wide x 12", 15", 18" and 24" deep shelves.  All other sizes are punched on 3" centers. Front and rear surfaces are punched for attaching accessories.  Tubular front edge is designed to protect against impact loads.  Used for uniformly distributed loads up to 800 pounds.

Hallowell H-Post High Capacity Closed Shelving Specifications (page 2)

Shelves –Extra Heavy Duty:  Formed with two ribs along the horizontal front and rear edge and are stamped on the top left corner for identification.  Class 2 shelves are fabricated from extra heavy gauge cold rolled steel and have roll formed rectangular tubular shaped front and rear edges with return bends resistance welded to the under side of shelf.  Welds are spaced 3" on center to provide maximum strength. Sides are triple flanged to form a channel.  All four corners are lapped and resistance welded to provide a rigid corner and add extra strength to the shelf.  Top surface shall be punched for attaching Shelf Dividers from left to right on 1-1/2" centers front and back, for 36" wide x 12", 15", 18" and 24" deep shelves.  All other sizes are punched on 3" centers.  Front and rear surfaces are punched for attaching accessories.  Tubular front edge is designed to protect against impact loads.  Used for uniformly distributed loads up to 1,200 pounds.

Shelf Clips:  Shall be a one piece compression clip drawn from 13 gauge cold rolled steel and zinc plated.  Fits snugly against side of post to hold shelves in place.  Provide four per shelf.

Universal Clip:  Fabricated from 16 gauge cold rolled steel formed into an angle with three slotted holes on one flange and one long slot in the other flange, which is also offset to fit in Beaded Front Post.  Provide when Swinging Doors are required.

Color for all painted components:  Posts and side panels shall be Hallowell #707 Marine Blue, Shelves and back panels shall be Hallowell #711 Light Gray.