Hallowell Bulk Storage Locker Specifications

General: Lockers shall be Hallowell Bulk Storage Lockers with all welded components as manufactured by Hallowell, 2823 West Orange Avenue, Apopka, FL 32703,1-866-566-0500, www.Hallowell-List.com. Quantities, sizes, and arrangements shall be as shown in the drawings.

The lockers shall be GREENGUARD GOLD CertifiedSM.

Locker Fronts: All doors, whether single or double tier, shall consist of 1/2” 16 gauge flattened expanded metal framed by 1-3/8” x 1” 12 gauge formed angles. Single tier doors shall have angle reinforcement across the center. Doors shall be pre-hung in frames at the factory, with all hinges, padlock hasps and door strikes securely welded in position. Door frames shall be fabricated from 1-1/2” x 1” 14gauge formed channels on the sides and 1-1/2” x 1” formed angles at top and bottom and include a 13” transom. All frames shall be pre-punched for ease of installation. All front assemblies are 1-1/2” deep.

Hinges: Hinges shall be heavy-duty 2-knuckle lift off hinge.

Vertical Dividers: Shall be 1/2” 16 gauge flattened expanded metal securely welded to 1-1/8” x 1-1/8”14 gauge formed angles. Channels shall be provided on both sides to support shelves and bottoms when specified. All vertical dividers are 1-1/2” less than nominal locker depth to allow for 1-1/2” deep door front assembly.

Repair Angles: 16 gauge 36” long repair angle is pre-punched on 2” centers to “heal” cuts in dividers or transom. Two bolts and clips are included with each angle.

Expanded Metal: Shall be of 1/2” 16 gauge flattened expanded metal for all doors, transoms, backs, vertical dividers, and tops.

Shelves: Where required, shall be of 22-gauge galvanized sheet steel with double bend at front and back. Provided as standard for all double tier lockers to separate upper tier from lower tier.

Backs: Where required, shall be of 1/2” 16 gauge flattened expanded.

Bottoms (optional): Where required, shall be of 22-gauge galvanized sheet steel with double bend at front and back.

Tops (optional): Shall be of 1/2” 16 gauge flattened expanded metal with a 16-gauge angle welded at rear. Provide mounting clips and bolts.

Finish: All fronts, vertical dividers, backs, and tops shall have one coat of 725 Dark Gray powder coat finish. Bottoms and shelves shall be galvanized sheet steel, unpainted.